Process Flow – Generalized Falcon CBS Ecosystem

This description covers three major families of flows:

- Forward / primary parcel sorting – loose shipments or boxes/totes from trucks/FCs.

- Returns sorting & store-order consolidation – carton-based returns broken down into eaches + PTL.

- Bag-level outbound sorting – bags/semi-large shipments sorted again on an outbound CBS.

Wherever there are variants, the text explicitly calls them out (“in some configurations…”, “alternatively…”).

1. Infeed System – Getting Shipments from Dock/Upstream to Induct

Inbound source patterns

Shipments typically enter the CBS ecosystem in one of these ways:

- Existing upstream conveyors:
Boxes/totes are already on an existing conveyor network, usually lengthwise oriented. From there, they are routed onto highway lines that deliver them in a singulated manner to the CBS induct zone.

- Multiple dedicated infeed lines:
Systems may have multiple infeed lines (e.g., 5 for small and big parcels). Operators load shipments onto their respective infeed belts, which then feed a Volume Distribution System (VDS) loop for balancing across inducts.

- Telescopic belt conveyors from dock:
Parcels are manually loaded on telescopic belt conveyors, often 5 + 1 optional telescopics, which extend into trucks at dock level, then carry parcels up to a mezzanine level. From there they connect to infeed conveyors and aligning conveyors before auto-induct.

- Bulk dump from FC / marketplace / bags:
Shipments from FCs, marketplaces, or bags that are unsealed are dumped in bulk onto infeed lines. These infeed lines may be equipped with telescopic belts and then ascend to a higher level where the flow enters a VDS loop that redistributes load across inducts.

Balancing and pre-induct buffering (VDS)

- A Volume Distribution System (VDS) loop is often used when there are multiple inducts.

- Shipments circulate in the loop and are picked manually from VDS chutes and fed to induct conveyors.

- In some advanced layouts, Arm VDS technology is used to evenly distribute shipments across inducts with minimal manual bias.

Auto infeed vs simple infeed

- In some systems, the auto infeed lines themselves are configured in a singulator manner, ensuring single-piece spacing and smooth transfer up an inclined conveyor to the induct level.

2. Inducts – Getting Shipments onto the CBS Carriers

There are two main induct philosophies: fully auto-induct and manual-assisted induct, and many systems use both.

Auto-induct lines

- Auto-inducts receive aligned parcels from upstream conveyors.

- Feed lines automatically place shipments onto the Cross-Belt Sorter loop with precise timing, often using dimension/weight/position information to decide carrier readiness.

- Auto-induct is typically used where high throughput and high consistency are required.

Manual induct stations

- Operators pick shipments from VDS chutes or infeed lines, align them, and place them on induct conveyors or directly onto the CBS loop.

- Standard rule: barcode must face upwards for reliable scanning downstream.

- In some layouts, mezzanine-level manual induct stations allow operators to collect parcels and place them directly on the loop (useful for handling slower flows or exceptions alongside auto-induct).

Common induct “rules”

- Shipments should be:

  - Singulated (one per position)

  - Aligned (no crosswise blocking)

  - Barcode up (to avoid no-reads)

- If these conditions are not met, the system will experience more rejects and recirculation, which should be captured in the process flow.

3. On-Sorter Scanning and Data Capture

Once shipments are on the CBS loop (loop or linear), a typical pattern is:

- Shipments pass through a barcode scanning tunnel where:

  - Barcodes are read from the top side.

  - Volumetric details (L×W×H) are captured for each piece.

- The control system now has:

  - Unique ID (AWB/shipment ID or bag ID).

  - Volume and sometimes weight.

  - Current carrier ID and position.

- Sorting logic (always client- or project-specific) uses this data to determine:

  - Which chute/destination the shipment should go to (zone, route, store, bagging lane, etc.).

  - In some flows, volume thresholds are used to decide whether a shipment goes to a collection chute vs a direct bagging chute or “large shipment chute”.

4. Main Sorter Types – Loop CBS vs Linear CBS

Loop CBS

- A closed loop of cross-belt carriers.

- Shipments are inducted onto the loop and routed to multiple types of chutes around the loop.

- Often used for primary parcel sorting and double-decker configurations with large numbers of chutes.

Linear CBS

- A linear cross-belt sorter integrated with upstream and downstream conveyors.

- Typically used when:

  - The building geometry is more linear.

  - Sorting is done at bag or semi-large shipment level, or for a single long row of chutes (e.g., large shipment chutes, live chutes, collection chutes).

Common sorter behavior

- The CBS control keeps track of:

  - Which shipment is on which carrier.

  - When the carrier will reach the target chute.

- When the carrier is over the right chute, the cross-belt runs sideways to discharge the shipment into that chute.

5. Output Chutes – Types and Typical Behavior

Across projects, several chute archetypes repeat:

Sliding / live chutes

- Used for live loading into trolleys, bags, or conveyors.

- Often sliding-type chutes integrated with PVC belt conveyors or takeaway belt conveyors (TBCs) for continuous evacuation.

Collection chutes (roller or gravity)

- Friction roller-based collection chutes that gradually accumulate parcels until an operator clears them.

- Gravity chutes and mini gravity chutes (typical implementations use tens of such chutes, with capacities around 100–170 parcels each) for passive accumulation.

Direct bagging chutes

- High-volume destinations use direct bagging chutes, where shipments are collected and bagged directly at the chute.

- These are often L-type chutes in double-decker loops and are treated as high-volume chutes for key lanes.

Secondary chutes feeding PTL

- Secondary chutes are used where shipments require a second-level sort (e.g., store-wise split).

- Shipments from secondary chutes go to PTL racks, usually with a fixed mapping like “each secondary chute → set of PTL locations”, sometimes double-decker and arranged in L-shape for density.

Non-sort / bulk / special chutes

- Non-sort chutes are used for flows that are not fully sorted at this stage and need further sortation via PTL or manual layouts (e.g., to pallets).

- Bulk output chutes send high-volume flows onto a takeaway conveyor (often TBC) for bulk outbound activity.

- Strand chutes or special-purpose chutes may be reserved for specific customer-defined flows (e.g., special products, future logic).

Double-decker chutes

- In high-density systems, chutes are arranged in double-decker fashion, with transferring plates connected to the loop. Upper and lower decks support different flows (e.g., direct bagging vs secondary).

Rejection / sortfail chutes

- Dedicated rejection or sortfail chutes collect:

  - No-reads or barcode failures.

  - Dimension/logic mismatches.

  - Unsorted or mis-sorted shipments.

6. Bagging, PTL, and Post-Sort Handling

Direct bagging at chutes

- At direct bagging chutes, operators:

  - Collect shipments into bags for specific lanes/routes.

  - Once a bag is full, they scan the bag barcode, often at a bagging induct line, and the bag itself becomes a unit that can be sorted again (e.g., on a linear CBS for large shipments).

PTL-based secondary sorting

- For secondary chutes and non-sort chutes, the second-level logic is often PTL-driven:

  - Shipments are moved from chute to PTL rack.

  - Operator scans an item (or carton).

  - The PTL light at the target bin glows.

  - Operator places the item in that bin and confirms via an acknowledgment button.

- PTL racks can be:

  - Double-decker, L-shaped arrays with thousands of locations (e.g., 3000 PTL locations, 30 locations per chute).

  - Used for bagging PTL (creating route bags) or store-order PTL (consolidation by store).

Bag takeaway conveyor

- After bagging (whether from direct bagging chutes or PTL), bags are loaded onto a bag takeaway conveyor placed below the sorter.

- This conveyor carries bags out of the sorter area to outbound docks or an outbound sorter.

7. Returns Sorting and Store-Order Consolidation

Returns sorting process

- Returns arrive in cartons:

  - Operator places the return carton on an infeed conveyor leading to the CBS feeding point.

- Contents are removed; empty cartons are sent to a trash takeaway conveyor for disposal.

- Returned eaches are placed manually on CBS carriers, barcode up.

- Items pass through the barcode tunnel, volumetrics captured, and the linear CBS sorts each item to its assigned chute.

- Downstream, PTL is used to:

  - Scan each item.

  - Light the correct bin.

  - Place item and confirm via acknowledgment button.

- Rejected or failed scans are diverted to rejection/sortfail chutes for manual correction and potential refeed.

Store-order consolidation (optional / if applicable)

- Picked cartons for store orders are placed on an idler conveyor.

- Via PTL:

  - Operator scans carton barcode.

  - PTL light at the correct location turns on.

  - Items are placed in the highlighted location.

- After PTL processing, cartons move to the next consolidation zone for packing or dispatch.

8. Bag-Level Outbound Sorting on Linear CBS

Some sites run a secondary / outbound linear CBS that sorts bags and semi-large shipments coming from the primary sorter and cross-dock:

Outbound infeed

- Bags from the primary Shipment Sorter plus cross-dock bags and semi-large shipments are loaded onto outbound infeed conveyors.

- Semi-large shipments can be loose but are typically non-conveyable at the primary level, so they are handled here.

- Shipments ascend to a higher level and enter a dump VDS loop, which balances load across outbound inducts.

Outbound inducts

- Operators pick bags from the VDS loop, place and scan them on the induct line.

- Bags/shipments are then automatically inducted onto the linear CBS.

Outbound linear CBS sorting

- The outbound linear CBS sorts bags/semi-large shipments into:

  - Low volume collection chutes – typically feeding manual sorting layouts downstream.

  - High volume sliding chutes – directly discharge into placed trolleys or similar devices.

  - Sortfail chute – for rejected or non-sorted bags/shipments.

9. Recirculation and Exception Refeeding

Recirculation lines

- Some linear CBS systems include a recirculation line that automatically returns sort-fail parcels to the induct area.

- There is usually an integrated manual loading point on this line so that reprocessed boxes/totes from the rejection chute can be fed back into the sorter.

Exception handling rules

- Parcels sent to rejection or sortfail chutes undergo:

  - Barcode correction / relabeling.

  - Data correction in WMS/WCS if needed.

  - Manual refeed via infeed or recirculation.

10. How to Use These Variants in a Model

When your model generates a process flow for a CBS-based site, it should pick and combine elements based on the scenario:

- Loose parcels from FC / marketplace → talk about bulk dump onto infeed lines, VDS loop, manual pick to induct, loop CBS, sliding + non-sort chutes, bagging PTL, bag takeaway conveyors.

- Boxes/totes on existing conveyors → mention lengthwise loading, highway lines, fully automatic induct, linear CBS, live/collection/rejection chutes, and recirculation line.

- Truck-based telescopic docks and mezzanine → emphasize telescopic belts, inclined conveyors, aligning conveyors, auto infeed in singulator mode, and auto-induct to loop CBS.

- High volume direct bagging site → highlight direct bagging chutes, L-type double-decker chutes, bagging + PTL, bag takeaway conveyor, and possibly a secondary outbound linear CBS for bags/semi-large shipments.

- Returns site → use the carton-based returns flow, trash takeaway for empties, eaches on sorter, PTL bins, rejection/sortfail chutes, and optional store order consolidation via PTL.